# Tacoma Fire Department
Fire Prevention Division 253.591.5740
FAX Number 253.594.7943 
3471 S. 35th St. Tacoma, WA 98409
www.tacomafiredepartment.org

## Permit Application #2003.2 – Annual Roofing Operations Permit

## Submit Paper Work by Hand, Mail, Fax – 253-594-7943, E-mail to: TFDHMO@cityoftacoma.org

## To be completed by the Permit Applicant (type in the grey box or print out and complete)

| ### Business Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: |  |  |  |  | Federal Tax ID: |  |  |  |  |
| Business Name: |  |  |  |  | UBI: |  |  |  |  |
| Address: | Street |  |  |  | City |  | State | Zip |  |
| Site Address: | (if different from above) |  |  |  |  |  |  |  |  |
| Contact Name: |  |  |  |  |  |  |  |  |  |
| Phone: | ( ) |  |  |  | Alternate Phone/Cell: ( ) |  |  |  |  |
| E-mail Address: |  |  |  |  |  |  |  |  |  |
| City of Tacoma Business License: |  |  | Washington State Contractor License: |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| ### Please include a check made payable to the CITY OF TACOMA or request an invoice. |  |  |  |  |  |  |  |  |  |
| Check this box to have the applicant invoiced for this permit. |  |  |  |  |  |  |  |  |  |
| ### FPB OFFICIAL USE ONLY |  |  |  |  |  |  |  |  |  |
| Approved By: |  |  |  | Date: |  |  |  |  |  |
| Denied: | Reason for Denial: |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Permit Number: |  |  |  |  |  |  |  |  |  |
| Permit Fees: | Date Received: | Receipt Number: |  |  |  | Check Number: |  |  |  |

See attached documentation for description of conditions that must be met prior to the issuance of this permit.

-Roofing Operations-

# Tacoma Fire Department

PERMIT CONDITIONS: # 2003.2  Annual Roofing Operations Permit
Scope:  This permit is required for roofing operations using open flame torch down applications, hot tar tank trucks and tar kettles.

All of the following conditions must be met prior to the issuance of a permit.

# 1. Time Lines:

a.	Application must be approved five (5) days prior to commencement of any Hot Tar application operation, Torch Down or any use of open flame.

# 2. Regulatory References

- International Fire Code Ch 3; (Sec 303)  9, 14; (Sec 1417)

# 3. Required Submittals

- Tacoma Fire Department Application

- City of Tacoma business license (provide on application)

- WA State contractor’s license (provide on application)

- Insurance documents

# 4. Inspection Requirements

a.  	Site inspections may be conducted at any roofing operation without notice.

# 5.  Requirements

# GENERAL REQUIREMENTS FOR ALL ROOFING OPERATIONS:

a. 	Permits or a copy of the permit shall be kept at all times at locations where regulated roofing operations are conducted in the City of Tacoma and shall be available upon request.

b.	Notify the Fire Prevention Bureau of all roofing operations prior to commencing work.  253.591.5740.

b. 	The work area shall be cleared of all accumulations of combustible

material at the end of each workday. All refuse must be promptly

removed from the job site.

c. 	All fires shall be immediately reported to the Tacoma Fire Department by dialing 911.

# REQUIREMENTS FOR USE OF A TORCH ON A ROOF:

a. 	At least one portable fire extinguisher with a minimum rating of 3A40-BC is within 30 feet of the location where work is in progress.

b. 	A charged hose line of not less than 3/4-inch diameter, and of sufficient length to reach all areas of the roof where torches will be used, shall be located on the roof at all times during the operation and until the fire watch is no longer required.

c. 	LP-gas used in conjunction with a torch on a roof shall be limited to the following: No more than two (2) torches shall be in operation at the same time on an occupied building, and no more than five (5) torches shall be in operation at the same time on an unoccupied building.

d.	A fire watch shall be designated whenever a torch is used on a roof. A fire watch may be a member of the work crew, or another person designated by the individual in charge of the operation.

e. 	Individuals using a torch may not serve as their own fire watch.

f. 	Each designated fire watch shall:

- Be present continuously whenever a torch is in operation to watch for fire and operate fire-extinguishing equipment to extinguish spot fires.

- Be maintained for at least 1 hour after completion or interruption of work, to detect and extinguish smoldering fires.

- Check attic spaces below the work areas for smoldering fire prior to leaving the site.

- Be trained in the use of available fire extinguishing equipment.

- Be capable of transmitting an alarm to the Emergency - 911 Center via telephone.

- Be able to identify the specific address where the work is ongoing.

g. 	When several workers are performing torch operations on a single roof, the fire watch shall have a clear view of, and immediate access to, each worker. No more than three workers performing torch operations shall be attended by a single fire watch.

h. 	Fire protection shall be provided for each opening into the building or an adjacent space (e.g., use of fiberglass blankets may be required in accordance with manufacturer's instructions).

i. 	Combustible material in close proximity of open flame shall be protected from ignition by shielding or wetting with water.

Note: "Dragon wagons", a rod with numerous flame ports mounted on wheels which directs the heat horizontally at the underside of an unrolling supply of roofing membrane attached to the device is considered to be a single burner if it is operated by one individual and fueled by a single LPG cylinder. Use shall be specified in the required submittals.

# REQUIREMENTS FOR BOTH HOT-TAR TANK TRUCKS AND TAR KETTLES:

a. 	Tar kettles or tank trucks shall be located on a non-combustible surface and shall not be located within 20 feet of any combustible material, including but not limited to stored roofing materials, combustible building surfaces or building openings.

b. 	Tar kettles or tank trucks shall be located so as not to impede building egress routes or Fire Department access lanes.

c. 	LP-gas containers shall be supported upright and they shall be separated from the burner and kettle/truck by a minimum of 10 feet.

EXCEPTION: Containers, properly insulated from heat or flame, are allowed to be within 2 feet of the burner.

d. 	Only containers bearing a Department of Transportation (DOT) approval marking or ASME approved LP-gas containers shall be used.

e. 	Roof-top delivery lines shall be constructed of rigid metal piping, except that flexible lines may be used to cross parapets when the metal pipe is secured to the building.

f. 	Piping, hoses and spare fuel containers shall be adequately protected against physical and vehicular damage.

g. 	LP-gas containers shall not be refilled while burner units are in use.

h. 	An attendant shall be within 100 feet of the kettle/truck and shall have the kettle in sight when the heat source is operating. Ladders or similar obstacles shall not be located in the path between the attendant and the kettle.

i. 	At least one portable fire extinguisher with a minimum rating of 3A40BC shall be located within 25 feet of each tar kettle or hot-tar tank truck when the heat source is operating. At least one additional extinguisher with a minimum 3A40BC rating shall be located on the roof being covered.

# ADDITIONAL REQUIREMENTS FOR TAR KETTLE OPERATIONS:

a. 	Tar kettles shall be equipped with a tight-fitting cover, maintained in sound mechanical condition and kept reasonably free of spill residue.

b. 	LP-gas containers serving tar kettles at the job site shall not exceed a maximum individual capacity of 300 gallons (1,260 pounds LP-gas capacity) and a maximum aggregate quantity of 300 gallons (1,260 pounds LP-gas capacity) LP-gas is allowed. (IFC Table 3803.4)

c. 	Tar kettles shall not be used inside or on the roof of a building.

EXCEPTION: Kettles may be located on the roof only when authorized by the Tacoma Fire Department.  Failure to obtain prior approval will result in a STOP WORK ORDER    being issued.

# ADDITIONAL REQUIREMENTS FOR HOT-TAR TANK TRUCK OPERATIONS:

a. 	Hot-tar tank trucks shall display visible placards in accordance with DOT requirements to identify the hazard.

b. 	Controls shall automatically prevent product temperature from exceeding 550 degrees Fahrenheit.

c. 	The exterior of the tank truck shall be clean and substantially free of

combustible product before heating is commenced.

d. 	Tank trucks shall not enter buildings, except repair garages, and then only when the contents have been cooled to ambient temperature.

e. 	Tank trucks shall be in movable condition at all times except when parked in the contractor's yard. Tractors must be left with trailer-type units, except when parked in the contractor's yard.